Wi-Fi Module 2

1. Brief about SplitMAC architecture and how it improves the AP's performance

Lower MAC functions are done by the AP.

APs perform functions such as client association, sending out beacons/probes, encrypting data, sounding, MU-MIMO/OFDMA, aggregation, ACKs

Upper MAC functions are moved and centralized into the controller

Functions such as RRM, QoS Management, Load balancing, traffic shaping/policing, mobility, authentication/security etc. are handled by the controller.

Performance improvement through splitMAC

- AP handles only time-critical tasks and heavy tasks moved to controller

- AP hardware becomes cheaper

- Faster roaming (controller controls handoff)

- Better RF management (RRM)

2. Describe about CAPWAP , explain the flow between AP and Controller

The WLC and the APs uses a protocol called CAPWAP (Control and Provisioning of Wireless Access Points) to communicate with each other.

Connection Process

- AP gets IP (DHCP)

- AP discovers WLC

- AP sends join request

- WLC authenticates AP

- CAPWAP tunnels created

Data flow

STA → AP → CAPWAP → WLC → Network → Internet

3. Where this CAPWAP fits in OSI model , what are the two tunnels in CAPWAP and its purpose

CAPWAP operates at Layer 3 (UDP-based)

Two tunnels are established in CAPWAP:

• Control Tunnel for all the control/management of the APs. All traffic on this tunnel is fully   encrypted.

• Data Tunnel for all the Data from/to APs pass through the controller

4. Whats the difference between Lightweight APs and Cloud-based APs

| Lightweight AP | Cloud AP |
| --- | --- |
| On-prem WLC | Controller on Cloud |
| Data traffic goes via WLC | Data traffic goes directly to internet |
| Centralized control(local) | Centralized control (cloud) |
| Needs WLC | Needs internet |

5. How the CAPWAP tunnel is maintained between AP and controller

- Uses keepalive, echo request/response messages

- AP periodically checks WLC availability

- If no response then tunnel drops and AP tries to rejoin controller

6.Whats the difference between Sniffer and monitor mode, use case for each mode

| Sniffer Mode : Packet capture - Captures raw wireless frames - Sends to WLC (or tools like Wireshark) - Used for debugging issues Use case: - Troubleshooting connectivity issues - Analyzing packet-level problems - Protocol debugging | Monitor Mode :Network monitoring & security - Scans environment continuously - Detects Rogue APs ,interference - Sends analytics/alerts to WLC Use case: - Security monitoring - Rogue AP detection - Network health analysis |
| --- | --- |

7.If WLC deployed in WAN, which AP mode is best for local network and how?

FlexConnect mode

- Works like LWAP normally

- If WAN/WLC fails then acts like autonomous AP

8. What are challenges if deploying autonomous APs (more than 50) in large network like university

- Manual config per AP

- No central management

- Difficult firmware updates

- Poor roaming

- VLAN scaling issues

- Large broadcast domains

- RF interference

9. What happens on wireless client connected to Lightweight AP in local mode if WLC goes down.

- Clients lose connectivity

- No new associations

- Existing sessions drop