Wifi Module 3

1.What are the different 802.11 PHY layer standards? Compare their characteristics.

| Standard | Technique | spectrum | features |
| --- | --- | --- | --- |
| 802.11 | FHSS, DSSS | 2.4Ghz |  |
| 802.11b | DSSS (CCK) | 2.4Ghz | First widely adopted standard. |
| 802.11a | OFDM | 5Ghz | OFDM for higher data rates. |
| 802.11g | OFDM | 2.4Ghz |  |
| 802.11n | OFDM | 2.4&5Ghz | MIMO and spatial multiplexing. |
| 802.11ac | OFDM | 5Ghz | MU-MIMO (Downlink) |
| 802.11ax | OFDMA | 2.4,5,6Ghz | OFDMA, MU-MIMO (UL/DL), and TWT. |
| 802.11be | OFDMA | 2.4,5,6Ghz | Multi-Link Operation (MLO) and U-SIG |

2. What are DSSS and FHSS? How do they work?

DSSS and FHSS are two types of Spread Spectrum technologies , it spreads the signal across a wider frequency band than necessary, this makes the signal harder to jam, more secure, and more resistant to interference from other devices

Direct Sequence Spread Spectrum :

spreads a  signal over a wide frequency band by multiplying it with a high-rate pseudo-noise  code

Frequency Hopping Spread Spectrum :

signal among many frequency channels in a pseudo-random, pre-determined sequence

This makes the signal harder to jam, more secure, and more resistant to interference from other devices

3. How do modulation schemes work in the PHY layer? Compare different modulation schemes and their performance across various Wi-Fi standards.

Modulation is the process of altering properties of a periodic waveform called carrier wave to carry information

| Modulation | Bits / Symbol | Data Rate | Wi-Fi Standards |
| --- | --- | --- | --- |
| BPSK | 1 | Very Low | 802.11a/g/n/ac/ax |
| QPSK | 2 | Low | 802.11a/g/n/ac/ax |
| 16-QAM | 4 | Medium | 802.11a/g/n/ac/ax |
| 64-QAM | 6 | High | 802.11n, 802.11ac |
| 256-QAM | 8 | Very High | 802.11ac |
| 1024-QAM | 10 | Extremely High | 802.11ax |
| 4096-QAM | 12 | Maximum | 802.11be |

4. What is the significance of OFDM in WLAN? How does it improve performance?

Splits a wide channel into many narrow subcarriers
          Sends data in parallel instead of one high-speed stream

Benefits:

- Hight data rate due to parallel transmission

- Better spectral efficiency due to subcarrier non overlapping

- Less susceptible to interference

5. How are frequency bands divided for Wi-Fi? Explain different bands and their channels.

| Feature | 2.4 GHz | 5 GHz | 6 GHz |
| --- | --- | --- | --- |
| Spectrum | Lowest 2.4 GHz to 2.483 GHz | Higher 5.150 GHz to 5.850 GHz | Highest 5.925 GHz to 7.125 GHz (~1.2 GHz spectrum) |
| Range | Highest | Medium | Lowest |
| Channels/Bandwidth | Few, overlapping/20Mhz | More non overlapping 20 / 40 / 80 / 160 MHz | Many (clean) 20 / 40 / 80 / 160 MHz |
| Speed | Lowest | Higher | Highest |

6. What is the role of Guard Intervals in WLAN transmission? How does a short Guard Interval improve efficiency?

Guard intervals  prevents overlap between symbols and handles multipath delay

Short Guard interval means smaller waiting time results higher throughput

7. Describe the structure of an 802.11 PHY layer frame. What are its key components?

PPDU (PLCP Protocol Data Unit) , the actual frame transmitted over air

Structure :

| Preamble | Header | Payload(PSDU) |
| --- | --- | --- |

Preamble : Synchronization ,Detect start of frame

Components :

L-STF (Short Training Field)

Frame detection

AGC (gain control)

Coarse frequency sync

L-LTF (Long Training Field)

Channel estimation

Fine frequency sync

L-SIG

Contains: Data rate ,Length of frame

PHY Header:

  - Modulation type

  - Data rate

  - Frame length

Payload (PSDU)

- Actual data from MAC layer

- Contains:

  - Management / Control / Data frames

8. What is the difference between OFDM and OFDMA?

| Feature | OFDM | OFDMA |
| --- | --- | --- |
| User Access | Single User (SU) | Multiple Users (MU) |
| Allocation | Time-based (users take turns) | Time & Frequency based (simultaneous) |
| Efficiency | Lower (wasteful for small packets) | Higher (optimizes Resource Units) |
| Latency | Higher (wait time for turn) | Much lower (parallel processing) |
| Best For | High-bandwidth single-stream data | Dense networks (IoT, offices, malls) |

9. What is the difference between MIMO and MU-MIMO?

| Feature | MIMO (SU-MIMO) | MU-MIMO |
| --- | --- | --- |
| Standard | Introduced with 802.11n (Wi-Fi 4) | Introduced with 802.11ac (Wi-Fi 5) |
| Simultaneous Users | 1 User at a time | Multiple Users (up to 8 in Wi-Fi 6/7) |
| Focus | Maximizing speed for a single device | Maximizing network capacity and efficiency |
| Requirement | Multiple antennas on both sides | Requires beamforming support |
| Traffic Direction | Uplink and Downlink | Historically Downlink; Wi-Fi 6 added Uplink |

10. What are PPDU, PLCP, and PMD in the PHY layer?

Physical Layer Convergence Procedure :

The PLCP is the upper sub-layer of the PHY layer.

Adds PHY layer headers to MAC frame including preamble and other information to create PPDU

Physical Medium Dependent

The PMD is the lower sub-layer of the PHY laye

Responsible for transmitting any bits it receives from the PLCP into the air using the antenna

Physical Layer Protocol Data Unit

The PPDU is the actual frame (packet) transmitted over the wireless medium.

11. What are the types of PPDU? Explain the PPDU frame format across different Wi-Fi generations.

| 802.11 Version | Format Name | Common Usage / Meaning |
| --- | --- | --- |
| 802.11a/b/g | Non-HT | Legacy / Non-High Throughput |
| 802.11n (Wi-Fi 4) | HT | High Throughput |
| 802.11ac (Wi-Fi 5) | VHT | Very High Throughput |
| 802.11ax (Wi-Fi 6) | HE | High Efficiency |
| 802.11be (Wi-Fi 7) | EHT | Extremely High Throughput |

12. How is the data rate calculated?

No.of Subcarrier * No.of Coded bits per subcarrier per stream * Coding * No.of spatial stream

Data rate   =

OFDM symbol duration + Guard interval Duration

The actual Throughput under ideal conditions is expected to be around 60-70% of the PHY data rate due to wifi overhead\