Wifi Module 4

- What is the significance of MAC layer and in which position it is placed in the OSI model

MAC layer is sublayer of Data Link Layer (OSI Layer 2)

Significance

Controls how devices access medium

Provides:

  - Framing

  - Addressing (MAC address)

  - Error detection (FCS)

  - Reliability (ACK + retry)

  - Medium access (CSMA/CA)

- Describe the frame format of the 802.11 MAC header and explain the purpose of each fields

| Frame control | Duration | Address 1 | Address 2 | Address 3 | Seq Control | Address 4 | QoS | HT Control |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Frame Control : It is a 2 bytes long field which defines the type of frame and some control information

  - Version: Current Protocol Version.

  - Type: management /control / data

  - Subtype: Indicates subtype such as Beacons, Probe Request, etc.

  - To DS: A 1-bit long field which, when set, indicates that the destination frame is for the DS (Distribution System).

  - From DS: A 1-bit long field which, when set, indicates the frame is coming from the DS.

  - More frag (More fragments): When set to 1, it means the frame is followed by other fragments.

  - Retry: A 1-bit long field. If the current frame is a retransmission of an earlier frame, this bit is set to 1.

  - Power Mgmt.: Set to 1, this field indicates that the station goes into power-save mode. If set to 0, the station stays active.

  - More data: A 1-bit long field used to indicate to the receiver that a sender has more data to send than the current frame.

  - WEP: A 1-bit long field which indicates that the standard security mechanism of 802.11 is applied.

  - Order: A 1-bit long field. If this bit is set to 1, the received frames must be processed in strict order.

- Duration/ID:  4-bytes long field which contains the value indicating the period of time in which the medium is occupied

- Address 1 to 4: 6-bytes long fields which contain standard IEEE 802 MAC addresses (48-bit each).

- Sequence Control: 16-bits long field which consists of 2 sub-fields: Sequence number (12 bits) and Fragment number (4 bits).

-   Data: A variable-length field which contains information specific to individual frames which is transferred transparently from a sender to the receiver(s).

-   CRC (Cyclic Redundancy Check): It is a 4-bytes long field which contains a 32-bit CRC error-detection sequence to ensure an error-free frame.

- Please list all the MAC layer functionalities in all Management, Control and Data plane

Management plane

- Beacon

- Probe

- Authentication

- Association / Reassociation

- Roaming

- Power management

Control plane

- RTS/CTS

- ACK / Block ACK

- NAV

- Channel access coordination

Data plane

- Data transfer

- QoS handling

- Aggregation

- Retransmission

- Explain the scanning process and its types in detail

Scanning is the first step for the station to join an AP’s network

Types:

Passive Scan : Client listen for beacon frames from AP
Active Scan : On demand connection through probe request and response

- Brief about the client association process

- Client scan and authenticate with AP and send association request

- If not authenticated , AP sends deauthentication frame from access point in response

- When the association request is granted, the access point responds with a status code of 0 (successful) and the Association ID (AID).

- Explain each steps involved in EAPOL 4-way handshake and the purpose of each keys derived from the process

- The Authenticator sends  ANonce to the Supplicant

- The Supplicant generates its own random value, the SNonce and send with MIC

- The Authenticator sends a message confirming the PTK is verified and sends GTK

- The Supplicant sends a final acknowledgment (MIC) to the AP

Keys derived

- PTK : unicast encryption

- GTK : broadcast traffic

- Describe the power saving scheme in MAC layer and explore on the types of Power saving mechanisms

AP buffers data when client enters sleep mode for power saving

Uses TIM in beacon

- Describe the Medium Access Control methodologies

Medium Access Control (MAC) methodologies are the set of rules and protocols used by the MAC sublayer to regulate how multiple devices share a single communication channel.

- DCF

The standard, decentralized method. It uses CSMA/CA with a binary exponential backoff timer.

- CSMA/CA

- Backoff

- NAV

- IFS

- PCF

Centralized method where an Access Point (AP) polls stations in a "contention-free" period.

- AP polling

- Contention-free period

- EDCA (QoS)

EDCA prioritizes traffic so that time-sensitive data, like a VoIP call or a video stream, gets to the medium faster than a background file download.

- Priority queues

- AIFS, CW, TXOP

- Brief about the Block ACK mechanism and its advantages

Block ACK acknowledge several frames in single block

Mechanism:

- TX check if RX supports Block ACK by sending Add Block ACK(ADDBA)

- RX can respond with a ADDBA response.

- After data transmission is completed the TX can send a Block Ack Request Frame to received the Block ACK.

- The block ACK frame will have bitmap that indicates which frames were not received and this allows the Tx to retransmit those frames.

- At the end of the session the Tx can close by sending a Delete Block Ack (DELBA) message

## Advantage

- Reduced ACK overhead

- Efficient retransmission

- Explain about A-MSDU, A-MPDU and A-MSDU in A-MPDU

A-MSDU

  - Multiple MSDUs → one MAC frame

  - Low overhead

  - no selective retransmission

  - more effecient

A-MPDU

  - Multiple MPDUs bundled each have separate MAC header with single PHY header

  - selective retransmission (Block ACK)

  - more reliable

Combined(A-MSDU in A-MPDU)

  - A-MSDU inside A-MPDU

  - best performance