Wi-Fi module 5

1. What are the key features of Wi-Fi 6, 6E and 7 and how do they differ from previous standards like Wi-Fi 5 (802.11ac)?

Wi-Fi 6 (802.11ax)

- OFDMA (multi-user efficiency)

- UL + DL MU-MIMO

- 1024-QAM

- TWT (power saving )

- BSS Coloring (interference reduction)

Wi-Fi 6E

- Adds 6 GHz band (1200 MHz spectrum)

- No legacy devices

- More channels (especially 160 MHz)

- Less interference

Wi-Fi 7 (802.11be)

- 320 MHz channels

- 4096-QAM

- Multi-Link Operation (MLO)

- Multi-RU + Puncturing

- Enhanced MU-MIMO

| Feature | Wi-Fi 5 | Wi-Fi 6 | Wi-Fi 6E | Wi-Fi 7 |
| --- | --- | --- | --- | --- |
| Band | 5 GHz | 2.4 + 5 | + 6 GHz | 2.4 + 5 + 6 |
| OFDMA | No | Yes | Yes | Yes |
| MU-MIMO | DL only | UL + DL | UL + DL | Enhanced |
| QAM | 256 | 1024 | 1024 | 4096 |
| Channel width | 160 MHz | 160 MHz | 160 MHz | 320 MHz |
| Key focus | Speed | Efficiency | Clean spectrum | Ultra-high throughput |

2. Explain the role of OFDMA in Wi-Fi 6 and how it improves network efficiency.

OFDMA divides a Wi-Fi channel into Resource Units (RUs) and allows:
 multiple users to transmit simultaneously

Advantages:

- Multi-user transmission and Parallel transmission

- AP-controlled scheduling, assigns RUs to users

- Supports both Downlink (AP to clients)and  Uplink (clients to AP)

- Reduces bandwidth wastage

3. Discuss the benefits of Target Wake Time (TWT) in Wi-Fi 6 for loT devices.

TWT lets a device and AP schedule exact wake-up times for communication. Device sleeps most of the time, wakes only when needed

Benefits in IoT devices:

Battery saving :   IoT devices send samller data need not to active always

Better Network Efficiency:  AP schedules devices for transmission

Scalability for IoT Networks :  Thousands of devices can be managed without congestion

4. Explain the significance of the 6 GHz frequency band in Wi-Fi 6E.

Huge spectrum : 6Ghz has massive 1200Mhz bandwidth compared to 2.4 and 5 Ghz

Wider channels : Supports upto 160Mhz channels enabling higher throughputs

No Backward Compatibility Overhead : Only Wi-Fi 6/6E devices are compatible

Less interference : clean spectrum due to absence of legacy devices

Best for dense environments: Less congestion due to more channels

5. Compare and contrast Wi-Fi 6 and Wi-Fi 6E in terms of range, bandwidth, and

interference.

| Feature | Wi-Fi 6 | Wi-Fi 6E |
| --- | --- | --- |
| Bands | 2.4 , 5 GHz | 2.4 , 5 GHz,6 GHz |
| Range | Higher | Lower |
| Bandwidth | Moderate | Very high |
| Interference | Moderate–High | Very low |
| Channels | Limited | Many |

6. What are the major innovations introduced in Wi-Fi 7 (802.11be)?

- 320 MHz Channel Width (higher data rate)

- 4096 QAM (higher throughput)

- Multi Link Operation : device can use multiple bands(2.4/5/6 Ghz ) simultaneously

- Multi RU Allocation

- Enhanced MU-MIMO

7. Explain the concept of Multi-Link Operation (MLO) and its impact on throughput and

latency.

MLO allows a device to use multiple frequency bands (2.4 GHz + 5 GHz + 6 GHz) simultaneously

Device creates multiple links with AP , data can be either split across links or sent on best available link

Modes of operation

1. Simultaneous transmission

Send data on multiple bands at same time and increases throughput

2. Enhanced Multi-Link Single Radio

Choose best band based on congestion interference and improves reliability

Impact on Throughput : Combines bandwidth of multiple bands

Impact on Latency: If one band is busy switch to another ,reducing delay and avoids retransmissions

8. What is the purpose of 802.11k and v, and how does it aid in roaming?

802.11k (Radio Resource Management) 
AP provides neighbor reports, allowing devices to identify the best available APs without scanning all channels, reducing latency, improving load balancing, and increasing battery life. By knowing which APs to scan, devices can switch APs faster

802.11v (Wireless Network Management)

Enables APs to advise clients on better APs to connect providing smarter roaming decisions and better load distribution

9. Explain the concept of Fast BSS Transition (802.11r) and its benefit in mobile

environments.

802.11r enables fast roaming by performing security key exchange before the actual handoff.

Problem in normal roaming

When moving to a new AP:

Authentication, Key exchange, Association causes delay, packet loss, call drops

With 802.11r

authentication + key setup in advance

Pre-compute:

PTK (Pairwise Transient Key)

Thus no full handshake needed when switching AP

Two methods

1. Over-the-Air : Client directly communicates with target AP

2. Over-the-DS : Communication happens via current AP

Benefits in mobile environments

Very low latency roaming : No delay during handoff

Seamless real-time applications (VoIP calls, video streaming,online gaming)

10. How do 802.11k/v/r work together to provide seamless roaming in enterprise networks?

802.11k

Gives neighbor AP list

reduces scanning time

802.11v

AP suggests best AP to move

Uses BSS Transition Management (BTM)

802.11r

Pre-authentication + key setup

fast handoff

Benefits

Very low roaming delay

No call drops

Efficient load distribution

Better user experience