Wifi Module 6

- What are the pillars of Wi-Fi security?

- Authentication : verifies identity

- Confidentiality  : encrypts data

- Integrity : ensures data not modified

- Explain the difference between authentication and encryption in WiFi security.

| Authentication | Encryption |
| --- | --- |
| Checks identity of sender/reciever | Secures data |
| Uses credentials like password | Uses keys to scramble data |
| Happens before access | Happens during data transfer |

- Explain the differences between WEP, WPA, WPA2, and WPA3.

| Feature | WEP | WPA | WPA2 | WPA3 |
| --- | --- | --- | --- | --- |
| Encryption | RC4 | RC4 + TKIP | AES-CCMP | AES-GCMP |
| Key Size | 40/104 bit | 128 bit | 128-bit | 128/192/256-bit |
| Cipher | Stream | Stream | Block | Block |
| Integrity | CRC-32 | MIC | CBC-MAC | SHA |
| Key Management | - | 4 way handshake | 4 way handshake | SAE |

- Why is WEP considered insecure compared to WPA2 or WPA3?

- Uses weak RC4 cipher

- Same key reused

- Small IV (24-bit) leading to repetition

- Easy to crack using packet analysis

- Why was WPA2 introduced?

WPA still used RC4 (weak)

WPA2 introduced:

- AES encryption

- Strong integrity (CCMP)

- Replay protection

- What is the role of the Pairwise Master Key (PMK) in the 4-way handshake?

PMK is root key for all session keys derived from PSK or EAP , it is used to generate PTK

- How does the 4-way handshake ensure mutual authentication between the client and the access point?

4 way handshake:

- AP sends Anonce

- Client sends Snonce

- Both generate same PTK

- Exchange + verify keys (MIC check)

By this process both client and AP can verify each other

- What will happen if we put a wrong passphrase during a 4Way handshake?

Wrong Passphrase generate wrong PMK and PTK which leads to MIC mismatch thus resulting in handshake failure

- What problem does 802.1X solve in a network?

Without 802.1X:

Same password for all users (PSK problem)

With 802.1X

- User-based authentication

- Centralized control (RADIUS server)

- No need to store keys on AP

- How does 802.1X enhance security over wireless networks?

Uses EAP authentication framework

Works with RADIUS server (AAA)

Supports:

- Certificates

- Username/password

Provides:

- Mutual authentication

- Dynamic key generation (PMK)

- Per-user access control